# Chapter 4 additional tables (parameter study, index coding, JPEG/JPEG 2000 reference)

Table 4.A. Effect of the block size (K = 64; mean of the eight grayscale images).

| Block size | PSNR (dB) | SSIM | CR | LBG iterations | LBG time (s) |
| --- | --- | --- | --- | --- | --- |
| 2×2 | 36.41 | 0.9446 | 8.35 | 178.6 | 0.4178 |
| 4×4 | 31.90 | 0.8785 | 27.09 | 145.0 | 0.2488 |
| 8×8 | 28.21 | 0.7907 | 38.33 | 102.0 | 0.1671 |

Table 4.B. Effect of the codebook size (4×4 blocks; mean of the eight grayscale images).

| K | PSNR (dB) | SSIM | CR | LBG iterations | LBG time (s) |
| --- | --- | --- | --- | --- | --- |
| 32 | 30.61 | 0.8527 | 36.67 | 106.9 | 0.1115 |
| 64 | 31.90 | 0.8785 | 27.09 | 145.0 | 0.2488 |
| 128 | 33.14 | 0.8979 | 20.14 | 184.1 | 0.5309 |

Table 4.C. Effect of index coding on the system compression ratio (4×4 blocks, K = 64; codebook of 1024 bytes included in all cases).

| Image | Fixed 8-bit index | Fixed 6-bit index | Delta + Huffman (proposed) | Gain over 6-bit (%) |
| --- | --- | --- | --- | --- |
| Lena | 15.06 | 19.69 | 23.50 | +19.3 |
| Boat | 15.06 | 19.69 | 26.54 | +34.8 |
| Cameraman | 15.06 | 19.69 | 28.88 | +46.7 |
| Peppers | 15.06 | 19.69 | 24.75 | +25.7 |
| Chest X-ray | 15.06 | 19.69 | 23.39 | +18.8 |
| Mammogram | 15.06 | 19.69 | 31.56 | +60.3 |
| Lung CT | 15.06 | 19.69 | 24.63 | +25.1 |
| Brain MRI | 15.06 | 19.69 | 33.49 | +70.1 |
| Mean | 15.06 | 19.69 | 27.09 | +37.6 |

Table 4.D. Rate-matched reference comparison with JPEG and JPEG 2000 (mean of the eight grayscale images).

| Method | CR | PSNR (dB) | SSIM |
| --- | --- | --- | --- |
| Proposed (LBG + Delta + Huffman + BF) | 27.09 | 31.90 | 0.8785 |
| JPEG | 27.02 | 34.81 | 0.8925 |
| JPEG 2000 | 27.31 | 37.24 | 0.9228 |

Table A.x. Per-image reference comparison with JPEG and JPEG 2000 (CR / PSNR in dB / SSIM).

| Image | Prop. CR | Prop. PSNR | Prop. SSIM | JPEG CR | JPEG PSNR | JPEG SSIM | J2K CR | J2K PSNR | J2K SSIM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Lena | 23.50 | 30.44 | 0.8431 | 23.37 | 33.14 | 0.8767 | 23.53 | 35.48 | 0.9059 |
| Boat | 26.54 | 29.65 | 0.8522 | 26.41 | 31.37 | 0.8705 | 26.53 | 33.64 | 0.9051 |
| Cameraman | 28.88 | 30.60 | 0.9031 | 29.23 | 34.01 | 0.9173 | 28.85 | 37.17 | 0.9446 |
| Peppers | 24.75 | 30.24 | 0.7875 | 24.92 | 32.24 | 0.8103 | 24.83 | 33.42 | 0.8310 |
| Chest_X-ray | 23.39 | 34.14 | 0.8890 | 22.82 | 36.88 | 0.9190 | 23.64 | 39.40 | 0.9470 |
| Mammogram | 31.56 | 36.73 | 0.9392 | 31.47 | 40.94 | 0.9541 | 32.59 | 43.58 | 0.9717 |
| Lung_CT | 24.63 | 28.90 | 0.8666 | 24.52 | 31.22 | 0.8743 | 25.06 | 33.13 | 0.9017 |
| Brain_MRI | 33.49 | 34.50 | 0.9474 | 33.39 | 38.66 | 0.9175 | 33.44 | 42.11 | 0.9756 |